Digital Watchdog® is a leading manufacturer of security and surveillance solutions, offering stunning image quality, advanced hardware capabilities, reliable customer support and the lowest total cost of deployment to the analog & IP megapixel surveillance markets. Located in Cerritos, CA, with manufacturing facilities in Seoul, Korea, Digital Watchdog® is committed to delivering powerful security solutions to its customers worldwide.

For additional information, contact:

Digital Watchdog®
	16220 Bloomfield Avenue,

Cerritos, California 90703 USA

Phone: +1 888 446-3593

Web: www.digital-watchdog.com

E-mail: dw-tech@digital-watchdog.com

MEGApix® Ai™ 32MP 4-SENSOR PANORAMIC IP CAMERA

DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 	Video Surveillance

28 21 00 	Surveillance Cameras

28 21 13	IP Cameras

Notes to Specifier:

1. Where several alternative parameters or specifications exist or where the specifier has the option of inserting text, such choices are presented in <bold text>.

2. Explanatory notes and comments are presented in colored text.

3. CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although the adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

4. MasterFormat 2014 Specification Category:

28 23 29 - Video Surveillance Remote Devices and Sensors

32MP 4-sensor panoramic camera with AI engine

- GENERAL

  - SUMMARY

    - The section features a 32MP high-definition 4-sensor multi-sensor panoramic IP video camera with an aluminum vandal-resistant, IP67-rated, and IK10 impact-resistant housing.

    - Product - A high-definition IP multi-sensor panoramic camera, based on H.265, H.264, and MJPEG compression, capable of quad streaming at 30 fps at resolutions up to 3856(H) X 2200(V) per sensor (4x), with Star-Light Plus™ color in near-total darkness technology and 4x 9.0mm fisheye lenses providing over 180° combined field of view.

    - The MEGApix Ai cameras feature an edge-based AI engine with real-time people detection, classification, and tracking, enabling intelligent video solutions.

    - Related Requirements

      - 27 15 01.13 – Video Surveillance Communications Conductors and Cables

      - 28 05 03 – Safety and Security Network Communications Equipment

      - 28 05 19 – Storage Appliances for Electronic Safety and Security

      - 28 06 20 – Schedules for Video Surveillance

      - 28 23 00 – Video Management System

  - REFERENCES

    - Abbreviations

      - AGC - Automatic Gain Control

      - ARP – Address Resolution Protocol

      - AWB  - Automatic White Balance

      - BLC – Backlight compression

      - DHCP - Dynamic Host Configuration Protocol

      - DNR – Digital Noise Reduction

      - DNS - Domain Name Server

      - DDNS – Dynamic Domain Name Server

      - fps - frames per second

      - FTP - File Transfer Protocol

      - GUI – Graphical User Interface

      - HLC – Highlight Compensation

      - HTTP - Hypertext Transfer Protocol

      - ICMP – Internet Control Message Protocol

      - IGMP - Internet Group Management Protocol

      - IP - Internet Protocol

      - JPEG - Joint Photographic Experts Group

      - MJPEG - Motion JPEG

      - NTP - Network Time Protocol

      - PoE - Power over Ethernet

      - QoS – Quality of Service

      - RARP – Reverse Address Resolution Protocol

      - RTP - Real-Time Transport Protocol

      - RTSP - Real-Time Streaming Protocol

      - SMTP - Simple Mail Transfer Protocol

      - TCP - Transmission Control Protocol

      - UDP - User Datagram Protocol

      - VMS - Video Management System

      - WDR – Wide Dynamic Range

    - Reference Standards

      - Network

        - IEEE - 802.3 Ethernet Standards

      - Video

        - ISO / IEC 14496 – MPEG-4

          - ISO / IEC 14496–10, MPEG-4 Part 10 (ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF – Profile S

      - Emissions

        - FCC-47 CFR Part 15 Class B

      - Environmental

        - ANSI / IEC60529 – Degrees of Protection Provided by Enclosures

        - International Electrotechnical Commission (IEC) – Ingress Protection Rating IP66

        - European standard EN 62262 — equivalent to International Electrotechnical Commission (IEC) – Impact Protection Rating IK10

  - SUBMITTALS

    - Product Data

      - Manufacturer's printed or electronic datasheets.

      - Manufacturer's installation and operation manuals.

      - Warranty documentation.

  - QUALIFICATIONS

    - The Manufacturer shall have at least five years of experience producing IP video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE, AND HANDLING

    - Deliver the camera in its manufacturer's original, unopened, and undamaged container, with identification labels intact.

    - Store the camera in an environment with a temperature range of -4°F to 122°F (-20°C to 50°C), protected from mechanical and environmental conditions as designated by the Manufacturer.

  - WARRANTY AND SUPPORT

    - The Manufacturer shall provide a 5-year warranty for the product, ensuring it is free from defects in material and workmanship.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Digital Watchdog, Inc.

16220 Bloomfield Avenue. Cerritos,

California, USA 90703 USA

Phone: (866) 446-3595

Web: www.digital-watchdog.com

E-mail:  dw-tech@digital-watchdog.com

    - Models: DWC-XMDF32H

    - Alternates: None

  - GENERAL DESCRIPTION

    - The high-definition multi-sensor panoramic IP camera ("IP camera") shall provide video performance capable of delivering selectable resolutions up to 3840(H) X 2160(V) pixels per sensor (4x sensors) at 30 frames per second (fps) with color in darkness technology, AI analytics capabilities, contained within an IP67-rated, IK10 impact-resistant vandal housing.

    - The camera shall include preinstalled AI deep learning analytics at the edge.

    - The IP camera shall possess the following characteristics:

      - H.265, H.264 and MJPEG compression

      - 32MP resolution (4x 8.48MP sensors)

      - Color in darkness technology (0.13 lux)

      - AI deep learning people detection, classification, and tracking

      - Four independent IP video streams (quad streaming, triple streaming per sensor)

      - Day/night operation with an IR cut filter

      - Combined panoramic field of view exceeding 180° with seamless image alignment and stitching

      - Integral motion detection

      - True Wide Dynamic Range

      - 3D digital noise reduction

      - Two-way audio via line input/output

      - PoE capable

      - Multicast or unicast capable

      - Local storage via 4x Micro SD, SDXC, or SDHC cards

      - Built-in web server

      - Dynamic DNS (DDNS) support

      - IP-67 rated

      - IK-10 impact-resistant

      - NDAA compliant

      - TAA-compliant

      - UL-listed

  - VIDEO

    - Imager

      - Sensor:	 	4x 8.48MP 1/1.8” Star-Light Plus™ sensor

      - Minimum illumination

        - Color mode: 		0.13 lux (color)

        - Black & white mode: 	0.007 lux

      - Scanning - Progressive

      - Image Control Settings shall be available for:

        - Exposure modes: 		automatic or manual

          - In manual exposure mode, configurable settings for:

            - Brightness

            - Shutter mode: 	automatic or manual - 1/15 to 1/32000

            - Digital slow shutter to decrease the shutter speed in low light

            - Anti-flicker shutter setting to address video flickering issues

            - Automatic Gain Control (AGC)

            - Wide Dynamic Range (WDR) level, 120dB

        - Frequency

        - Image sharpness

        - Image mirror or flip

        - Digital noise reduction

      - Lens

        - Lens type: 4x 9.0mm, F2.6 fisheye lens.

        - Horizontal field of view (HFOV): 46.6° (x4), over 180° combined

        - Vertical Field of view (VFOV): 27.1°

        - Privacy zones:  16 programmable privacy masks

        - True Wide Dynamic Range, 120dB

        - Shutter speed:  1/15 sec to 1/32000

    - Video Streams

      - The IP camera shall support four (4) configurable video streams (one per sensor), selectable from H.265, H.264 and MJPEG, with triple streaming per channel (3rd stream MJPEG only).

      - Each of the camera’s four video channels shall support the following properties:

        - Compression Type and Resolution:

          - H.265: 3840x2160, 2560x1440, 1920x1080, 1280x720, 704x576, 704x480, 640x360, 352x288, 352x240 (per sensor, x4)

          - H.264: 3840x2160, 2560x1440, 1920x1080, 1280x720, 704x576, 704x480, 640x360, 352x288, 352x240 (per sensor, x4)

          - MJPEG: 1920x1080, 1280x720, 768x432, 704x576, 704x480, 640x360, 352x288, 352x240 (per sensor, x4)

      - Frame rate:	0 – 30 fps

    - Streaming Capability: 	4x Triple Streaming (3rd stream MJPEG only)

    - Motion Detection – The IP camera shall be able to detect motion within user-defined areas of the video image.

      - Configuration settings shall be available for sensitivity and dwell time.

    - Storage and Recording

      - The IP camera shall have onboard SD card storage via four (4) local card slots.

        - Card Type:	Micro SD/SDHC/SDXC Class 10

        - Up to 1TB microSD / FAT32

      - Local recording on the SD card shall commence upon loss of network connectivity, based on a pre-programmed schedule.

      - The local SD storage shall be backed up to alternate media without removing the SD card from the camera.

    - ONVIF – Video streams shall be capable of supporting the ONVIF protocol, profile S.

    - Other

      - Single Images - The IP camera shall support the JPG file format for image screenshots and exports.

      - Discovery - The Manufacturer shall offer a discovery program to identify all devices manufactured by the Manufacturer that are connected to the network.

      - Emergency Alarm – The IP camera shall enable the transmission of a video clip of configurable duration to up to five server locations.

      - Access - The IP camera shall permit up to five users to access the camera simultaneously.

  - AUDIO

    - The IP camera shall provide two-way audio via one line input and one line output.

    - The IP camera shall have one audio input line (1.0Vrms, 3K ohms).

    - Compression and Sampling Rate:  G.711 ulaw.

  - NETWORK

    - Connectivity: 1000Base-T (1Gbps) Ethernet with RJ-45 connector

    - Protocols supported

      - Transmission Control Protocol (TCP), Internet Protocol (IP) v4, User Datagram Protocol (UDP)

      - Configuration: Dynamic Host Configuration Protocol (DHCP)

      - Web services: Hypertext Transfer Protocol (HTTP)

      - Network services: Domain Name System (DNS), Network Time Protocol (NTP), Internet Control Message Protocol (ICMP), Simple Network Management Protocol (SNMP)

      - Media: Real-Time Transport Protocol (RTP), Real-Time Streaming Protocol (RTSP)

    - DDNS – The IP camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

    - Security

      - The IP camera shall support IP address filtering, allowing users to enter a list of allowed or blocked IP addresses for viewing video and configuring camera settings.

      - The IP camera shall provide digest authentication, as well as ID and password protection.

      - The IP camera shall provide three user access levels with password protection.

  - EMBEDDED ANALYTICS

    - Deep Learning people tracking

    - Zones and lines, tamper, metadata, intrusion, line crossing, counting, counting lines, appearing, disappearing, stopping, entering, exiting, direction, tailgating, dwell time, logical rules.

  - ADDITIONAL FEATURES

    - Auxiliary Inputs and Outputs

      - Input: Alarm sensor (contact closure)

      - Output: Relay

      - Event trigger: Motion alarm, network loss, temperature anomaly, illegal login, schedule, sensor detection

    - System Information

      - The IP camera system settings shall be exported as a separate file.

      - The IP camera shall maintain an access log of the system and motion-triggered events.

        - The log shall be exportable to an Excel spreadsheet file.

  - CAMERA SOFTWARE

    - The IP camera shall have a built-in web server that supports browser-based configuration using Internet Explorer, Google Chrome, Mozilla Firefox, and Apple Safari.

    - The software GUI shall allow access to camera information and all primary software functions, including:

      - Camera network address

      - Configuration

      - Stream Control

      - Screen snapshot, print and export

      - Start and stop recording

    - The Manufacturer shall offer a video viewer and configuration to implement the following actions:

      - Camera discovery

      - Configuration

        - video parameters

        - events and notifications

          - motion-related

          - emergency

        - camera network parameters

        - SD card storage recording management

        - image capture, export, and print

      - Viewer - view video streams through the web browser

      - Image print and export

      - Instant recording and playback

      - Alerts

        - e-mail setup

        - Define web addresses for notifications

      - System

        - firmware upgrade

        - reset to factory default

        - set date, time, and NTP server synchronization

        - user access control

        - View and export camera settings

        - View system logs

  - ELECTRICAL

    - Power

      - Sources

        - DC12V

        - PoE+ (high-power injector included)

      - Power Consumption

        - DC12V: 			max 31.6W, 2.63A

        - PoE: 			max 35.2W, 733mA

      - Connectors

        - Ethernet: 			RJ-45 connector

        - External power (DC12V): 	2-conductor power to a terminal block

  - MECHANICAL AND ENVIRONMENTAL

    - Housing Material: 			Aluminum vandal housing, polycarbonate lens cover

    - Configuration: 			Multi-sensor panoramic camera

    - Environmental Rating: 		IP-67

IK-10

    - Dimensions (D x H):  		8.26" x 5.15" (210 x 131mm)

    - Weight: 				6.6 lbs (3 kg)

    - Temperature:

      - Operating: 			-22° F to 122° F (-30° C to 50° C)

    - Humidity:				10 - 90%, non-condensing

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor before camera installation.

  - INSTALLATION

    - The Contractor shall follow all Manufacturer-published installation procedures and guidelines.

    - Before permanent installation of the system, the system shall be factory tested in conditions simulating the final installed environment

      - A report indicating successful test results shall be produced.

  - STORAGE

    - The IP camera hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION