Digital Watchdog® is a leading manufacturer of security and surveillance solutions, offering stunning image quality, advanced hardware capabilities, reliable customer support and the lowest total cost of deployment to the analog & IP megapixel surveillance markets. Located in Cerritos, CA, with manufacturing facilities in Seoul, Korea, Digital Watchdog® is committed to delivering powerful security solutions to its customers worldwide.

For additional information, contact:

Digital Watchdog®
	16220 Bloomfield Avenue,

Cerritos, California 90703 USA

Phone: +1 866 446-3593

Web: www.digital-watchdog.com

E-mail: dw-tech@digital-watchdog.com

MEGApix® Ai™ 3MP PTZ IP CAMERA

DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 	Video Surveillance

28 21 00 	Surveillance Cameras

28 21 13	IP Cameras

Notes to Specifier:

1. Where several alternative parameters or specifications exist or where the specifier has the option of inserting text, such choices are presented in <bold text>.

2. Explanatory notes and comments are presented in colored text.

3. CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although the adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

4. MasterFormat 2014 Specification Category:

28 23 29 - Video Surveillance Remote Devices and Sensors

3MP PTZ camera with AI engine and 40x optical zoom

- GENERAL

  - SUMMARY

    - The section features a 3MP high-definition PTZ (pan-tilt-zoom) IP video camera with a 40x optical zoom lens, endless 360° pan, and an aluminum die-cast, IP66-rated, IK10 impact-resistant PTZ housing.

    - Product - A high-definition PTZ IP camera, based on H.265, H.264, and MJPEG compression, capable of quad streaming at up to 60fps at resolutions up to 2304(H) x 1296(V), with Star-Light Plus color in near-total darkness technology and a 6.5mm-260mm vari-focal motorized lens with 40x optical zoom and auto-focus, offering 360° endless pan and a 100° tilt range.

    - The MEGApix Ai cameras feature an edge-based AI engine with real-time object and people detection, classification, and tracking, across up to 40 configurable detection zones, enabling intelligent video solutions.

    - Related Requirements

      - 27 15 01.13 – Video Surveillance Communications Conductors and Cables

      - 28 05 03 – Safety and Security Network Communications Equipment

      - 28 05 19 – Storage Appliances for Electronic Safety and Security

      - 28 06 20 – Schedules for Video Surveillance

      - 28 23 00 – Video Management System

  - REFERENCES

    - Abbreviations

      - AGC - Automatic Gain Control

      - ARP – Address Resolution Protocol

      - AWB  - Automatic White Balance

      - BLC – Backlight compression

      - DHCP - Dynamic Host Configuration Protocol

      - DNR – Digital Noise Reduction

      - DNS - Domain Name Server

      - DDNS – Dynamic Domain Name Server

      - fps - frames per second

      - FTP - File Transfer Protocol

      - GUI – Graphical User Interface

      - HLC – Highlight Compensation

      - HTTP - Hypertext Transfer Protocol

      - ICMP – Internet Control Message Protocol

      - IGMP - Internet Group Management Protocol

      - IP - Internet Protocol

      - JPEG - Joint Photographic Experts Group

      - MJPEG - Motion JPEG

      - NTP - Network Time Protocol

      - PoE - Power over Ethernet

      - QoS – Quality of Service

      - RARP – Reverse Address Resolution Protocol

      - RTP - Real-Time Transport Protocol

      - RTSP - Real-Time Streaming Protocol

      - SMTP - Simple Mail Transfer Protocol

      - TCP - Transmission Control Protocol

      - UDP - User Datagram Protocol

      - VMS - Video Management System

      - WDR – Wide Dynamic Range

    - Reference Standards

      - Network

        - IEEE - 802.3 Ethernet Standards

      - Video

        - ISO / IEC 14496 – MPEG-4

          - ISO / IEC 14496–10, MPEG-4 Part 10 (ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF – Profile S

      - Emissions

        - FCC-47 CFR Part 15 Class B

      - Environmental

        - ANSI / IEC60529 – Degrees of Protection Provided by Enclosures

        - International Electrotechnical Commission (IEC) – Ingress Protection Rating IP66

        - European standard EN 62262 — equivalent to International Electrotechnical Commission (IEC) – Impact Protection Rating IK10

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic datasheets.

      - Manufacturer’s installation and operation manuals.

      - Warranty documentation.

  - QUALIFICATIONS

    - The Manufacturer shall have at least five years of experience producing IP video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE, AND HANDLING

    - Deliver the camera in its manufacturer’s original, unopened, and undamaged container, with identification labels intact.

    - Store the camera in an environment with a temperature range of -22°F ~ 140°F (-30°C ~ 60°C), protected from mechanical and environmental conditions as designated by the Manufacturer.

  - WARRANTY AND SUPPORT

    - The Manufacturer shall provide a 2-year warranty for the product, ensuring it is free from defects in material and workmanship.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Digital Watchdog, Inc.

16220 Bloomfield Avenue, Cerritos,

California, USA 90703 USA

Phone: (866) 446-3595

Web: www.digital-watchdog.com

E-mail:  dw-tech@digital-watchdog.com

    - Models: DWC-XPZA03Mi

    - Alternates: None

  - GENERAL DESCRIPTION

    - The PTZ IP camera (“IP camera”) shall provide video performance capable of delivering selectable resolutions up to 2304(H) x 1296(V) pixels at up to 60 frames per second (fps) with color in darkness technology, AI analytics capabilities, and 40x optical zoom, contained within an IP66-rated, IK10 impact-resistant PTZ housing.

    - The camera shall include preinstalled AI deep learning analytics at the edge.

    - The IP camera shall possess the following characteristics:

      - H.265, H.264 and MJPEG compression

      - 3MP resolution (3MP 1/1.8” CMOS sensor)

      - Color in darkness technology (0.005 lux (color), 0 lux (B/W))

      - 40x optical zoom and 16x digital zoom

      - 360° endless pan range and 100° (-10° to 90°) tilt range

      - High-speed PTZ with up to 500 presets, 8 swing patterns, 4 patterns, and 8 groups

      - AI deep learning object and people detection, classification, and tracking with up to 40 configurable detection zones

      - Quad streaming IP video (one primary and three secondary streams)

      - Day/night operation (TDN, Auto, day (color), night (B/W))

      - Integral Smart IR illuminator providing a 1148 ft (350 m) distance

      - True Wide Dynamic Range

      - Smart DNR™ 3D digital noise reduction

      - Two-way audio via line input/output

      - PoE and 24V AC capable

      - Multicast or unicast capable

      - Built-in web server

      - Dynamic DNS (DDNS) support

      - IP66 rated

      - IK-10 impact-resistant

      - NDAA compliant

      - TAA-compliant

  - VIDEO

    - Imager

      - Sensor:	3MP 1/1.8” CMOS sensor, 2304(H) x 1296(V) active pixels

      - Minimum illumination

        - Color mode: 		0.005 lux (color)

        - Black & white mode: 	0.0 lux (B/W)

      - Scanning - Progressive

      - Image Control Settings shall be available for:

        - Exposure modes: 		automatic or manual

          - In manual exposure mode, configurable settings for:

            - Brightness

            - Shutter mode: 	automatic or manual - 1/1 to 1/100,000

            - Digital slow shutter to decrease the shutter speed in low light

            - Anti-flicker shutter setting to address video flickering issues

            - Automatic Gain Control (AGC)

            - Wide Dynamic Range (WDR): True WDR

        - Frequency

        - Image sharpness

        - Image mirror or flip

        - Digital noise reduction

      - Lens

    - Pan, Tilt, Zoom (PTZ) Function

      - Pan range: 360° endless rotation

      - Pan speed: Max 250°/sec; manual speed 0.065°/sec to 250°/sec

      - Tilt range: 100° (-10° to 90°)

      - Tilt speed: Max 130°/sec; manual speed 0.065°/sec to 130°/sec

      - Optical zoom: 40x; digital zoom: 16x

      - Presets: 500 programmable presets

      - Swing: 8 programmable swing patterns

      - Pattern: 4 programmable patterns

      - Group: 8 groups

      - Auto parking and auto tracking

        - Lens type: 6.5mm-260mm, F1.4 vari-focal lens with motorized zoom and auto-focus, 40x optical zoom / 16x digital zoom.

        - Horizontal field of view (HFOV): 1.6°~64.1°

        - Vertical field of view (VFOV): 0.83°~33.4°

        - IR illumination distance:  1148 ft (350m) range

        - True Wide Dynamic Range

        - Shutter speed:  1/1 sec to 1/100,000

    - Video Streams

      - The IP camera shall support quad streaming: one primary stream (H.265 or H.264) and three secondary streams (H.265, H.264, or MJPEG).

      - Each stream shall support the following properties:

        - Compression Type and Resolution:

          - H.265, H.264, MJPEG: 352x240 ~ 2304x1296 (selectable resolutions)

      - Video Bitrate:	Primary Stream: 32Kbps ~ 16Mbps

Secondary Stream: 32Kbps ~ 4Mbps

      - Frame rate:	Max 60fps @ 2304(H) x 1296(V)

    - Streaming Capability: 	Quad streaming: Primary H.265/H.264; Secondary x3: H.265, H.264, MJPEG

    - Motion Detection – The IP camera shall be able to detect motion within user-defined areas of the video image.

      - Configuration settings shall be available for sensitivity and dwell time.

    - ONVIF – Video streams shall be capable of supporting the ONVIF protocol, profile G, S, T and M.

    - Other

      - Single Images - The IP camera shall support the JPG file format for image screenshots and exports.

      - Discovery - The Manufacturer shall offer a discovery program to identify all devices manufactured by the Manufacturer that are connected to the network.

      - Emergency Alarm – The IP camera shall enable the transmission of a video clip of configurable duration to up to five server locations.

      - Access - The IP camera shall permit up to five users to access the camera simultaneously.

  - AUDIO

    - The IP camera shall provide two-way audio via one line input and one line output.

    - The IP camera shall have one audio input line (1.0Vrms, 3K ohms).

    - Compression and Sampling Rate:  G.711.

  - ALARM I/O

    - The IP camera shall provide 4 alarm inputs (contact closure), 2 relay outputs for interfacing with external alarm and access control devices, and 1x RS-485 port for PTZ/auxiliary device control.

    - Alarm trigger sources shall include: Motion alarm, sensor input, client disconnection, user-defined event.

    - Upon alarm trigger, the camera shall be able to perform the following notification actions:

      - File upload (image) via e-mail or FTP

      - E-mail notification

      - Recording to SD/SDHC/SDXC card

      - Alarm relay output

  - NETWORK

    - Connectivity: 10/100Base-T Ethernet with RJ-45 connector

    - Protocols supported

      - Transmission Control Protocol (TCP), User Datagram Protocol (UDP), Internet Group Management Protocol (IGMP), Internet Control Message Protocol (ICMP)

      - Configuration: Dynamic Host Configuration Protocol (DHCP)

      - Web services: Hypertext Transfer Protocol (HTTP), HTTPS

      - Network services: Domain Name System (DNS), Network Time Protocol (NTP), Simple Network Management Protocol (SNMP), Simple Mail Transfer Protocol (SMTP), File Transfer Protocol (FTP), Universal Plug and Play (UPnP), WS-Discovery, Zero Configuration

      - Media: Real-Time Transport Protocol (RTP), Real-Time Streaming Protocol (RTSP), Real-Time Messaging Protocol (RTMP), Secure Reliable Transport (SRT), MPEG Transport Stream (MPEG-TS)

    - DDNS – The IP camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

    - Security

      - The IP camera shall support IP address filtering, allowing users to enter a list of allowed or blocked IP addresses for viewing video and configuring camera settings.

      - The IP camera shall provide digest authentication, as well as ID and password protection.

      - The IP camera shall provide three user access levels with password protection.

  - EMBEDDED ANALYTICS

    - Intelligent Video Analytics (AI-Based) – The IP camera shall provide the following configurable AI-based video analytics on the visible imaging channel:

      - AI-based object tracking with person and vehicle classification.

      - Virtual line crossing, counting lines, and virtual zones (enter/exit/appear/disappear/stopped).

      - Direction detection, tailgating detection, intrusion detection, and dwell (loitering) analytics.

      - Tamper detection, object metadata generation, and configurable logical rules.

      - Configurable for up to 40 separate detection zones and lines.

  - ADDITIONAL FEATURES

    - System Information

      - The IP camera system settings shall be exported as a separate file.

      - The IP camera shall maintain an access log of the system and motion-triggered events.

        - The log shall be exportable to an Excel spreadsheet file.

  - CAMERA SOFTWARE

    - The IP camera shall have a built-in web server that supports browser-based configuration using Microsoft Edge, Google Chrome, Mozilla Firefox, and Apple Safari, compatible with Windows, iOS, and Linux operating systems.

    - The software GUI shall allow access to camera information and all primary software functions, including:

      - Camera network address

      - Configuration

      - Stream Control

      - Screen snapshot, print and export

      - Start and stop recording

    - The Manufacturer shall offer a video viewer and configuration to implement the following actions:

      - Camera discovery

      - Configuration

        - video parameters

        - events and notifications

          - motion-related

          - emergency

        - camera network parameters

        - PTZ control (pan, tilt, zoom, presets, patterns, swings, and auto tracking)

        - image capture, export, and print

      - Viewer - view video streams through the web browser

      - Image print and export

      - Instant recording and playback

      - Alerts

        - e-mail setup

        - Define web addresses for notifications

      - System

        - firmware upgrade

        - Reset to factory default

        - Set date, time, and NTP server synchronization

        - User access control

        - View and export camera settings

        - View system logs

  - ELECTRICAL

    - Power

      - Sources

        - 24V AC

        - PoE (injector included)

      - Power Consumption

        - 24V AC: 			max 48.62W, 2.02A

        - PoE: 			max 39.05W, 813mA

      - Connectors

        - Ethernet: 			RJ-45 connector

        - External power (24V AC): 	2-conductor power to a terminal block

  - MECHANICAL AND ENVIRONMENTAL

    - Housing Material: 			Aluminum die-casting PTZ camera housing

    - Configuration: 			PTZ dome/bullet camera

    - Environmental Rating: 		IP-66

IK-10

    - Dimensions:  			8.89” x 14.3” (226 x 364mm)

    - Weight: 				12.72 lbs (5.77 kg)

    - Temperature:

      - Operating: 			-22° F to 140° F (-30° C to 60° C)

    - Humidity:				0 - 90% RH, non-condensing

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor before camera installation.

  - INSTALLATION

    - The Contractor shall follow all Manufacturer-published installation procedures and guidelines.

    - Before permanent installation of the system, the system shall be factory tested in conditions simulating the final installed environment

      - A report indicating successful test results shall be produced.

  - STORAGE

    - The IP camera hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION