Digital Watchdog is a leading manufacturer of security and surveillance solutions, offering stunning image quality, advanced hardware capabilities, reliable customer support and lowest total cost of deployment to the analog & IP megapixel surveillance markets. Located in Tampa, FL with manufacturing facilities in Seoul, Korea, Digital Watchdog is committed to delivering powerful security solutions to its customers worldwide.

For additional information, contact:

# Digital Watchdog®

# 16220 Bloomfield Avenue.

# Cerritos, California USA 90703 USA

# Phone: (866) 446-3595

# Web: www.digital-watchdog.com

# E-mail: dw-tech@digital-watchdog.com

HIGHDEFINITION PTZ DOMEVIDEO CAMERA

# DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

2820 00 	Electronic Surveillance

28 23 00 	Video Surveillance

28 23 29	Video Surveillance Remote Devices and Sensors

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented incoloredtext.

HIGHDEFINITION PTZ DOME VIDEO CAMERA

- GENERAL

  - SUMMARY

    - Section includes a1080p highdefinition IP PTZ video camerawithindoor ceiling or flush mount dome housing.

    - Product - A highdefinition IP PTZ video camera,  based on H.264, MPEG-4 and MJPEG compression, capable of streaming 30 fps at resolutions up to1920 x 1080, with dual streaming capability in anindoor dome housing.

## Related Requirements

      - 27 20 00 Data Communications

      - 28 23 13 Video Surveillance Control and Management Systems

      - 28 23 16 Video Surveillance Monitoring and Supervisory Interfaces

      - 28 23 19 Digital Video Recorders and Analog Recording Devices

      - 28 23 23 Video Surveillance Systems Infrastructure

  - REFERENCES

    - Abbreviations

      - AGC - Automatic Gain Control

      - ARP – Address Resolution Protocol

      - AWB  - Automatic White Balance

      - BLC – Back light compression

      - DHCP - Dynamic Host Configuration Protocol

      - DNR – Digital Noise Reduction

      - DNS - Domain Name Server

      - DDNS –Dynamic Domain Name Server

      - fps - frames per second

      - FTP - File Transfer Protocol

      - GUI – Graphical User Interface

      - HLC – Highlight Compensation

      - HME – High Masking Exposure

      - HTTP - Hypertext Transfer Protocol

      - ICMP – Internet Control Message Protocol

      - IGMP - Internet Group Management Protocol

      - IP - Internet Protocol

      - JPEG - Joint Photographic Experts Group

      - MJPEG - Motion JPEG

      - MPEG - Moving Pictures Experts Group

      - NTP - Network Time Protocol

      - PoE - Power over Ethernet

      - PPPoE - Point-to-Point Protocol over Ethernet

      - QoS – Quality of Service

      - RARP – Reverse Address Resolution Protocol

      - RTP - Real-Time Transport Protocol

      - RTSP - Real-Time Streaming Protocol

      - SMTP - Simple Mail Transfer Protocol

      - SNMP – Simple Network Management Protocol

      - TCP - Transmission Control Protocol

      - UDP - User Datagram Protocol

      - UPnP – Universal Plug and Play

      - VMS - Video Management System

      - WDR – Wide Dynamic Range

    - Reference Standards

      - Network

        - IEEE - 802.3 Ethernet Standards

      - Video

        - ISO / IEC 14496 – MPEG-4

          - ISO / IEC 14496–10, MPEG-4 Part 10 ( ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF–Profile S

      - Emissions

        - FCC-47 CFR Part 15 Class B

      - Environmental

        - ANSI / IEC60529 – Degrees of Protection Provided by Enclosures

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

      - Warranty documentation

  - QUALIFICATIONS

    - Manufacturershall have a minimum of five years’ experience inproducing IP video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the camera in a temperature environment of -10° C to 50° C (14° F to 122° F), protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY AND SUPPORT

    - Manufacturer shall provide a limited 2 yearwarrantyfor the product to be free of defects in material and workmanship.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Digital Watchdog®

16220 Bloomfield Avenue.

Cerritos, California USA 90703 USA

Phone: (866) 446-3595

Web: www.digital-watchdog.com

E-mail: dw-tech@digital-watchdog.com

    - Model		MPTZ5XFM

Digital Watchdog also offers the MPTZ5X surface mount version of this product.

    - Alternates: None

  - GENERAL DESCRIPTION

    - The High Definition IP PTZ Dome Camera (“PTZ Dome camera”) shall provide video performance capable of providing selectable resolutions up to1920 x 1080 pixels at 30 frames per second (fps).

    - The PTZ domecamera shall possess the following characteristics:

      - H.264, MPEG-4 and MJPEG compression

      - 2.1 megapixels

      - two independent IP video streams (dual streaming)

      - day/night operationwith IR cut filter

      - 12 x digital zoom, 5 x optical zoom

      - Wide Dynamic Range (WDR)

      - 16 privacy masking regions

      - integral motion detection

      - 3D digital noise reduction

      - two-way audio

      - alarm input and output

      - PoE capable

      - multicast or unicast capable

      - local storage via Micro SD or SDHC card

      - Dynamic DNS (DDNS) support

      - recessed ceiling or flush mount

      - built-in fan

  - VIDEO

    - Imager

      - Sensor:	1/2.8" Sony CMOS

      - Minimum illumination

        - Color mode: 		0.1 lux (F3.5 – 30IRE)

        - Black & white mode:	0.01 lux (F3.5 – 30IRE)

      - Scanning - Progressive

      - Image Control Settings shall be available for:

        - Automatic white balance (AWB)

          - Selectable for Auto, Indoor, Preset, Auto External, or Manual

            - Manual mode to provide the ability to configure red offset, blue offset and warmth or coolness of light.

        - Exposure  modes: 		automatic or manual

          - In manual exposure mode, configurable settings for:

            - brightness

            - shutter mode:	automatic or manual - 1/30 to 1/60000

            - digital slow shutter up to 8x to decrease shutter speed in low light

            - AGC:		0 - 41 dB maximum

            - Backlight compensation (BLC)

            - Highlight compensation (HLC)

            - WDR level:		100 dB minimum

            - Highlight Masking Exposure (HME)

        - Day and night settings, to allow configuration for Day (color), Night (Black and White), or Automatic.

        - Frequency

        - Image sharpness

        - Chroma

        - Image mirror or flip

        - Digital noise reduction

        - Privacy zone definition, up to 16 zones

      - Lens:		5 - 25 mm variable, F3.5 – F3.7, remote auto-focus

    - Video Streams

      - The PTZ dome camera shall support two configurable video streams, each of which may have the following properties:

        - Resolution:

          - Stream 1:	320 x 240 (4 x 3) to 1920 x 1080 (16:9)

          - Stream 2:	 320x176 to 960x544 (16:9)

        - Compression type:

          - H.264

          - MPEG-4

          - MJPEG

        - Bit rate:	1.8 Mbps – 8 Mbps

        - Frame rate:	0 – 30 fps

    - Motion Detection – The PTZ dome camera shall have the ability to detect motion within user defined areas of the video image.

      - Configuration settings shall be available for sensitivity and dwell time.

    - Storage and Recording

      - The PTZ dome control shall have onboard SD card storage.

        - Card type:	Micro SDHC Class 10

        - Capacity:	4 GB – 32 GB

      - Local recording on the SD card shall commence upon loss of network connectivity, based on a pre-programmed schedule.

      - The local SD storage shall have the ability to be backed up to alternate media without removal of the SD card from the camera.

    - ONVIF – Video streams shall be capable of supporting ONVIF protocol, profile S.

    - Pan Tilt

      - Pan Range:	360o

      - Tilt Range:	90o

      - Presets:	up to 255

      - Sequences:	up to 4

      - Tours:		up to 8

        - entries per tour:	20

      - Speed:		120o per second

    - Other

      - Single Images - The PTZ dome camera shall support jpg file image screenshot and export.

      - Discovery - Manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

      - Emergency alarm – The PTZ dome camera shall allow sending of a video clip of configurable duration to up to five server locations.

      - Access- The PTZ dome camera shall permit up to five users to simultaneously access the camera.

      - Alarm/Sensors – The PTZ dome camera shall have one alarm/sensor input and one alarm/control output.

  - AUDIO

    - The PTZ dome camera shall provide one line level input and one line level output.

    - Compression and Sampling Rate

      - G.711 (8 kHz)

      - AAC (48 kHz)

  - NETWORK

    - Connectivity: 100 BASE-TX Ethernet with RJ-45 connector

    - Protocols supported

      - Transmission Control Protocol(TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

      - Configuration: Dynamic Host Configuration Protocol (DHCP)

      - Web services: Hypertext Transfer Protocol (HTTP)

      - Network services: Domain Name System (DNS), NetworkTime Protocol (NTP), Internet Control Message Protocol (ICMP), Simple Network Management Protocol (SNMP), Universal Plug and Play (UPnP)

      - Media: Real-Time Transport Protocol (RTP), Real-Time Streaming Protocol (RTSP)

      - Multicast: Internet Group Management Protocol (IGMP)

      - Notifications: File Transfer Protocol (FTP), Simple MailTransfer Protocol (SMTP)

      - Remote Access: Point-to-Point Protocol over Ethernet)(PPPoE)

    - Flow Control –The PTZ dome camera shall support TCP and Layer 3 QoS protocols to control network congestion.

    - DDNS – ThePTZ dome camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

    - Security

      - The PTZ domecamera shall support IP address filtering whereby users can enter a list of allowed or blocked IP addresses for viewing video and configuring camera settings

      - The PTZ dome camera shall provide threelevels of user access with password protection.

  - ADDITIONAL FEATURES

    - System Information

      - The system settings of the PTZ dome camera shall be exportable as a separate file.

      - The PTZ dome camera shall maintain an accessible log of system and motion-triggered events.

        - The log shall be exportable to an Excel spreadsheet file.

  - CAMERA SOFTWARE

    - The PTZ dome camera shall have a built in web server which supports browser-based configuration using Internet Explorer, Google Chrome, Mozilla Firefox, Apple Safari.

    - The software GUI shall allow access to camera information and all primary software functions, including:

      - Camera network address

      - Configuration

      - Stream control

      - Screen snapshot, print and export

      - Start and stop recording

    - The Manufacturer shall offer video viewer and configuration to implement the following actions:

      - Camera discovery

      - Configuration

        - video parameters

        - events and notifications

          - motion related

          - emergency

        - camera network parameters

        - SD card storage recording management

        - image capture, export, and print

      - Viewer - view video streams through the web browser

      - Image print and export

      - Instant record and playback

      - Alerts

        - e-mail setup

        - define web addresses for notifications

      - System

        - firmware upgrade

        - reset to factory default

        - set date, time, and NTP server synchronization

        - user access control

        - view and export camera settings

        - view system logs

  - ELECTRICAL

    - Power

      - Sources

        - 12 VDC

        - PoE (IEEE 802.3af. 802.3at Class 3) - 48 VDC

      - Power Consumption:		8 W, 670 mA maximum

      - Ethernet: 			RJ-45connector

      - External power (12 VDC):	2-conductor power to terminal block

      - Audio and alarm:		8 conductor cable to terminal strip

  - MECHANICAL AND ENVIRONMENTAL

    - Housing Material: 		polycarbonate

    - Configuration:		PTZ dome

    - Dimensions (D x H): 	4.3 in. x 5.8 in. (110mm x 149 mm)

    - Temperature:

      - Operating:		-10° C to 55° C (14° F to 131° F)

    - Humidity:			0 - 90%, non-condensing

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).Network configuration shall be tested and qualified by the Contractor prior to camera installation.

  - INSTALLATION

    - Before permanent installation of the system, the system shall be factory tested in conditions simulating the final installed environment

      - A report indicating successful test results shall be produced.

  - STORAGE

    - The PTZ dome camera hardwareshall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION

6